| Sprint 1 Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hardware Assembly - Hardware and Integration Tests To ensure components are being handled correctly by the ESP8266 microcontrollers for each node |  |  |  |  |
| Node 1 – Sensor Node |  |  |  |  |
| Test ID | Feature | Test Action | Expected Result | Pass/ Fail |
| TC - 01 | Ultrasonic sensor (water level sensor) | Place an object exactly 10cm away from the sensor. | The serial monitor prints a reading of 10cm (± 1cm) |  |
| TC - 02 | Ultrasonic sensor (water level sensor) | Take a reading from the ultrasonic sensor. Fill the water container with 5cm more water. | The serial monitor shows a reading of 5cm increase in water level |  |
| TC - 03 | DHT11 – Temperature and Humidity Sensor (Temperature) | Check current room temperature. Print the DHT11 temperature reading to serial monitor | The current room temperature and DHT11 temperature should be the same (± 2°C) |  |
| TC - 04 | DHT11 – Temperature and Humidity Sensor (Temperature) | Print the DHT11 temperature reading to serial monitor. Then turn on the heating element of fan heater 1m away from the sensor for 2 minutes. | Sensor readings should gradually increase over the two minutes. |  |
| TC - 05 | DHT11 – Temperature and Humidity Sensor (Humidity) | Check current room humidity. Print the DHT11 humidity reading to serial monitor | The current room humidity and DHT11 temperature should be the same (± 2°C) |  |
| TC - 06 | DHT11 – Temperature and Humidity Sensor (Humidity) | Print the DHT11 Humidity reading to serial monitor. Then turn on the heating element of fan heater 1m away from the sensor for 2 minutes. | Sensor readings should gradually decrease over the two minutes. |  |
| TC-07 | Capacitive Soil Moisture Sensor | Insert capacitive soil moisture sensor in the soil of a dry, not watered plant | Serial Monitor displays a high reading |  |
| TC-08 | Capacitive Soil Moisture Sensor | Insert capacitive soil moisture sensor in the soil of a healthy watered plant | Serial Monitor displays a normal reading |  |
| Node 2 – Mitigation Node |  |  |  |  |
| TC - 09 | Servo Motor (floodgate) | The floodgate (through servo) should initially be disabled | The servo should be at position 0° (off/ deactivated) |  |
| TC - 10 | Servo Motor (floodgate) | Flash a command to force the servo to open | The servo should be at position 90° (enabled/ activated) |  |
| TC - 11 | Pump relay (for irrigation system) | Submerge the pump in water and enable the pump | The pump relay should pump water through the tubes and out into the desired container |  |